COMPUTER ENGINEERING WORKSHOP PROJECT REPORT

KEYPAD BASED DOOR LOCK SYSTEM USING ARDUINO UNO

Semester Project Report

Submitted by:

Name: Aneeza Gul
Department: Computer System Engineering
Semester: 1st Semester

Submitted to:

Course Instructor: Sir Irfan
Course Title: Computer Engineering Workshop

Institution Name:

Sukkur Institute of Business Administration

Date of submission:

01/01/2026

ABSTRACT:

Security is one of the most important concerns in modern society. Traditional mechanical locks are increasingly being replaced by electronic security systems due to their higher reliability and flexibility. This project presents the design and implementation of a Keypad Based Door Lock System using Arduino Uno.

The system allows authorized access through a 4×4 keypad by entering a predefined password. The user must press the ‘#’ key to start entering the password, and upon entering the correct code “1234”, the door is unlocked using a servo motor. The door remains open for 4 seconds and then automatically locks again to enhance security.

This project demonstrates the practical application of embedded systems, microcontroller programming, and basic automation techniques. The system is low-cost, easy to implement, and suitable for homes, offices, and laboratories.

- INTRODUCTION:

In recent years, electronic security systems have gained significant importance due to the increasing need for safety and controlled access. Conventional locks can be easily duplicated or damaged, whereas electronic locks provide enhanced security and control.

The Keypad Door Lock System is an embedded system that replaces traditional keys with a password-based authentication mechanism. The system is controlled by an Arduino Uno, which processes the user input from the keypad and controls a servo motor acting as the locking mechanism.

This project is designed as a beginner-level embedded system suitable for first-semester students, helping them understand the integration of hardware and software.

- PROJECT OBJECTIVES:

The main objectives of this project are:

- To design a secure password-based door locking system

- To learn Arduino Uno programming

- To interface a 4×4 keypad with Arduino

- To control a servo motor for locking and unlocking

- To implement auto-lock functionality after a fixed time

- To enhance understanding of embedded system concepts

- SYSTEM OVERVIEW

The system works on a simple principle:

- User presses ‘#’ to activate password entry mode

- User enters the password 1234

- User presses ‘#’ again to submit the password

- If the password is correct:

  - Servo rotates to unlock the door

  - Door remains unlocked for 4 seconds

  - Door automatically locks again

- If the password is incorrect:

  - Access is denied

- Pressing ‘D’ locks the door manually

- Pressing ‘*’ clears the entered input

4. HARDWARE COMPONENTS

| Component | Description |
| --- | --- |
| Arduino Uno | Main microcontroller |
| 4×4 Keypad | Password input device |
| Servo Motor | Door lock actuator |
| Jumper Wires | Electrical connections |
| Breadboard | Circuit assembly |
| Power Supply | USB / 5V |

  - SOFTWARE REQUIREMENTS

. Arduino IDE

. Keypad Library

. Servo Library

6. CIRCUIT DESCRIPTION

- The 4×4 keypad is connected to Arduino digital pins for rows and columns

- The servo motor is connected to digital pin 10

- Arduino processes keypad inputs and controls the servo accordingly

- Serial Monitor is used for debugging and system status messages

7. WORKING PRINCIPLE

Initially, the system keeps the door locked. The user must press the ‘#’ key to begin entering the password. Once the password “1234” is entered and confirmed by pressing ‘#’ again, Arduino verifies the input.

If the password matches:

- The servo rotates to 0°, unlocking the door

- A timer starts using millis()

- After 4 seconds, the servo rotates back to 90°, locking the door

This non-blocking timing technique ensures smooth system operation without delays.

8. FLOW CHART

- Start

- Initialize Arduino and servo

- Wait for key press

- If # → start password entry

- Enter digits

- Press # to submit

- If password correct → unlock door

- Wait 4 seconds

- Auto-lock door

- End